InVEST stormwater retention model

Authors: Perrine Hamel (perrine.hamel@ntu.edu.sg), Ben Janke (janke024@umn.edu)

Updated: Dec 2020 BDJ

## Overview

The model calculates annual stormwater retention volume and the associated water quality benefits. The value of the retention service may be calculated using a replacement cost of stormwater infrastructure.

## Model description

- Estimate stormwater retention and infiltration

The model requires values of the annual runoff coefficients (RC), and optionally the infiltration ratios (IR), for each LULC type in the biophysical table (See Table 1). RCs are defined as the ratio between annual runoff and annual precipitation, a value between 0 and 1, while the infiltration ratio is the ratio between annual infiltration to groundwater and annual precipitation. See “Input guidance” for how to find these values.

For each LULC x, the stormwater retention coefficients R is calculated as:

Rx=1-RCx

[Eq. 2-1]

Table 1. Example of “Biophysical Table” with runoff coefficients (RC_), infiltration ratios(IR_), and event mean concentrations (EMC_) of phosphorus (P) and nitrogen (N) for each soil group (A,B,C,D).

| Description | lucode | EMC_P | EMC_N | RC_A | RC_B | RC_C | RC_D | IR_A | IR_B | IR_C | IR_D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Background | 0 | 0 | 0 | 0.00 | 0.10 | 0.19 | 0.35 | 0.110 | 0.049 | 0.021 | 0.007 |
| Unclassified | 1 | 0 | 0 | 0.00 | 0.10 | 0.19 | 0.35 | 0.110 | 0.049 | 0.021 | 0.007 |
| High Intensity Developed | 2 | 0.275 | 2.33 | 0.76 | 0.77 | 0.77 | 0.79 | 0.011 | 0.006 | 0.003 | 0.001 |
| Med Intensity Developed | 3 | 0.345 | 2.53 | 0.55 | 0.58 | 0.60 | 0.65 | 0.039 | 0.021 | 0.011 | 0.003 |
| Low Intensity Developed | 4 | 0.393 | 2.34 | 0.30 | 0.35 | 0.39 | 0.48 | 0.072 | 0.039 | 0.020 | 0.005 |
| Open Space Developed | 5 | 0.296 | 2.85 | 0.09 | 0.16 | 0.22 | 0.35 | 0.099 | 0.053 | 0.027 | 0.007 |
| Cultivated Land | 6 | 0.506 | 3.44 | 0.00 | 0.09 | 0.16 | 0.30 | 0.110 | 0.057 | 0.028 | 0.007 |
| Pasture/Hay | 7 | 0.053 | 1.25 | 0.00 | 0.09 | 0.16 | 0.30 | 0.110 | 0.057 | 0.028 | 0.007 |
| Grassland | 8 | 0.053 | 1.25 | 0.00 | 0.09 | 0.16 | 0.30 | 0.110 | 0.057 | 0.028 | 0.007 |
| Deciduous Forest | 9 | 0.105 | 1.23 | 0.00 | 0.08 | 0.15 | 0.28 | 0.110 | 0.062 | 0.032 | 0.008 |
| Evergreen Forest | 10 | 0.105 | 1.23 | 0.00 | 0.08 | 0.15 | 0.28 | 0.110 | 0.062 | 0.032 | 0.008 |
| Mixed Forest | 11 | 0.105 | 1.23 | 0.00 | 0.08 | 0.15 | 0.28 | 0.110 | 0.062 | 0.032 | 0.008 |
| Scrub/Shrub | 12 | 0.053 | 1.25 | 0.00 | 0.09 | 0.16 | 0.30 | 0.110 | 0.057 | 0.028 | 0.007 |
| Open Water / Wetland | 13 | 0 | 0 | 0.00 | 0.00 | 0.00 | 0.00 | 0.000 | 0.000 | 0.000 | 0.000 |

Based on the LULC and hydrologic soil group rasters, the model assigns the stormwater retention coefficients (Ri) to each pixel i. Next, the model computes VR , the retained volume (m3/yr) as:

VR=0.001∙Pi ×Ri×pixel.area

[Eq. 2-2]

where Pi is annual precipitation (mm/yr) and pixel.area is the pixel area in m2

Optionally, if infiltration ratios have been defined by the user, the model assign these values to each pixel i (based on the LULC and soil hydrological group rasters), and computes Vinf, the infiltrated volume (m3/yr):

Vinf,i=0.001∙Pi×IRi×pixel.area

[Eq. 2-3]

Where IRi is the annual infiltration ratio (Table 1).

Optional: Adjust Retention Coefficient for (assumed) Directly-Connected Impervious

Retention on a given pixel, at least when considered at an aggregated sub-watershed scale, should be affected by retention on upstream and downstream (or at least surrounding) pixels. The current InVEST NDR model takes this into account through a flowpath delineation and retention length framework, which is not as feasible in an urban drainage network because of how storm drains alter surface flow paths. A simple modification to retention was tested in which the retention coefficient, Ri,j, for the pixel located at i,j was adjusted upwards based on the retention value of all 8 surrounding pixels. The adjustment, Ci,j,  was applied to the un-retained runoff from a given pixel, as follows:

[Eq. 2-4]

where Radji,j is the adjusted retention coefficient, and the adjustment factor Ci,j has one of the following values:

- 0, if any of the 8 adjacent pixels are considered directly-connected impervious (medium- or high-intensity urban land cover classes (Table 1)), or the pixels coincide with a street, as determined from an overlay with a road line layer;

- Rmean, the average retention coefficient of the surrounding 8 pixels.

In other words, no additional retention is provided by surrounding land if the pixel is considered directly-connected, i.e. is adjacent to dense urban land use or to roadways, which in most urban areas are directly connected to the drainage network by ditches or sub-surface pipes. Otherwise, the pixel’s retention coefficient is increased proportional to the retention provided by its neighboring pixels.

Considerations: This approach produces additional retention in large pervious areas such as agriculture, parks, golf courses, and cemeteries, and should provide improved accuracy in more developed watersheds, where the assumption of direct connectedness of roadways is more probable due to the prevalence of piped, curb-and-gutter roadways. However, the approach may “over-correct” (under-predict) retention for more rural watersheds, especially those with primarily ditched roadways, where the assumption of direct connection of roadways may fall short due to additional retention that might occur in ditches due to increased residence time and/or infiltration. Further, this approach will be affected by the resolution of the input rasters, as it only considers adjacent cells rather than a distance (the latter approach being used by NDR). A simple test of the adjustment for gauged watersheds in the Minneapolis-St. Paul, MN metropolitan area is shown in Section 2.5.

Additional Note (from BDJ, Dec 10, 2020): Per initial discussions with Rich on this model, he had suggested using a “retention length” or radius around a given pixel (cell i,j in Eq. 2-4), over which to consider addition retention vs. impervious connectivity (rather than simply using adjacent cells in the raster). This would remove the dependency on raster size of the retention modification calculation. This pixel-size dependency has no basis in real world conditions. I’ve included below a transcript of an email exchange between Rich and I, just for reference.

“This design doc looks good to us. We have some thoughts about the "Optional: Adjust Retention Coefficient for (assumed) Directly-Connected Impervious" section -- namely, why do we only consider the neighboring pixel to determine connectivity? As soon as you tie a biophysical process to a pixel you have pixel size affecting a biophysical process that's not based on pixels, i.e. pixel size rather than real distance (what if your pixels were 30 meters vs 1 meter but everything else was held constant?). Would it make more sense to consider a certain distance downhill from a current pixel? LIke if there's a drain within 50 meters "not downhill" then the current pixel also drains to that pixel?” [Richard Sharp, email dated Sep 3, 2020]

“The pixel size / connectivity issue is a fair critique, and one that Perrine and I had grappled with in putting this together. It works well for the 30m NLCD data because that length scale seems appropriate for urban drainage (and it's similar to the 20m we used for retention length in applying the NDR model to the Twin Cities in the first phase of the Golf project). Of course, for 1m data that's not far enough, creating a pixel size dependency, as you point out. I think what I was trying to avoid was having a "retention length" parameter for the runoff model, because then we might just end up re-creating the NDR model, or most of it anyway. I think we also assumed that 30m resolution or similar was more likely to be available than 1m, but that's not really a good excuse.

I suppose in an ideal world we'd have a retention length parameter, and have the model check, for each pixel, if there is a downslope road or high-density urban pixel within that distance, which would identify that pixel as "directly-connected." I wasn't able to think of a straightforward way to do this in ArcMap initially, so I just went with the neighboring pixel approach.” [Reply by Ben Janke, email dated Sep 3, 2020]

“Sounds like you thought it through pretty well. I don't know what the biophysical basis for it is, but if you're okay with the 30m being a direct connection without a retention length parameter then I don't see why you'd need to worry an arbitrary 30m distance needing a retention length parameter if it was over 30 1 meter pixels. How about we program a custom distance cutoff for distance to urban drainage it the prototype and you can fiddle with it and see if you like it? If you don't want users to fiddle with it we can hard code that to 30m or we can expose it as a custom parameter -- whatever you like.” [Reply by Richard Sharp, email dated Sep 3, 2020]

- Calculate water quality benefits of stormwater retention

The potential water quality impact of stormwater retention is determined as the pollutant mass associated with retained stormwater, i.e. the amount of pollutant load avoided. The annual avoided pollutant load, in kg/yr, is calculated for each pixel i as the product of runoff volume (m3/yr) and the event mean concentration (EMC) of a pollutant, in mg/L:

Loadi=1000∙VR,i∙EMC

[Eq. 2-5]

EMCs for N and P are assigned to land use classes using the biophysical look-up table (Table 1).

It should also be noted that users may add other stormwater pollutants (such as sediment, metals, or organic compounds) to the model by providing EMC values for those pollutants in the biophysical table.

- Valuation of stormwater retention service (optional)

A review of the most common valuation methods for the stormwater retention service can be found in a working paper by Sahl et al. (2015). If stormwater regulations exist, the user can assess the value of stormwater retention with the target retention volume as a reference. The economic value can be assessed if the average value of retention device ($/m3) is available.

Retention.cost=PR×VR

[Eq. 2-6]

Where PR is the replacement cost of stormwater retention ($/m3). For example, Simpson and McPherson (2007) estimate this to be $1.59/m3 for urban areas in the SF Bay Area.

The model can output potential groundwater recharge volume (Vinf from Eq. 2-3), which may also serve as a valuation of retention. However, the model does not currently estimate the pollutant load associated with this recharge volume, as sub-surface transport and transformation of pollutants is not implemented in the model.

d) Aggregation at the watershed scale (optional)

If users provide a watershed (or sewershed) shapefile, the model will aggregate the runoff and load rasters to compute:

- Average stormwater retention R (-) (average of R values, [Eq. 2-1])

- Total retention volume, VR (m3) (sum of VR values, [Eq. 2-2])

- Total load retained (kg/yr) (sum of retained loads, [Eq. 2-5])

- (Optional) Total potential recharge volume, Vinf (sum of recharge values, [Eq. 2-3])

- (Optional) Total Replacement Cost ($) (sum of retention costs, Eq. 2.5)

Summary for software implementation:

The logical flow of the model is a follows:

- Create a layer “LULC x Soil Hydrologic Group” as a combination of the two separate layers

- Create the retention ratio raster (based on the LULC x Soil Hydrologic Group raster and the biophysical table). Optional: also create the infiltration ratio raster

- Create the retention volume and retention value layers (based on the retention ratio raster and precipitation raster). Optional: also create the infiltration volume raster

- Aggregate results in the watershed shapefiles.

## Summary of inputs

| Name | Type | Description |
| --- | --- | --- |
| LULC | Raster | Land Use / Land Cover classification |
| P | Raster | Annual precipitation, in mm/yr |
| Road Centerlines (optional) | Shapefile | Road centerlines, used to adjust retention coefficient for pixels next to directly-connected impervious |
| Subwatershed or watershed of interest (optional) | Shapefile | Spatial units of interest (results will be aggregated at this scale). These can be obtained with DelineateIT |
| Biophysical Table (Example: Table 1) | Decimals | Table with columns: - lucode: LULC code - RC_A,…,RC_D: runoff coefficients (adimensional) corresponding to the LULC type and soil group A, …., D - IR_A, …, IR_D (optional): infiltration ratios (adimensional), corresponding to the LULC type and soil group A, …, D - EMC_N, (and EMC_P, optional): Event mean concentration (mg/L) for each pollutant (N or P); Default values for Nitrogen and Phosphorus are specified from the U.S. NSQD (bmpdatabase.org/nsqd.html) and other references (see Section 2.5) |
| Soil hydrologic group | Raster | Raster with values equal to 1, 2, 3, 4, corresponding to soil hydrologic group A, B, C, or D, respectively (i.e., high infiltration capacity to minimal infiltration capacity). Potential source: https://daac.ornl.gov/SOILS/guides/Global_Hydrologic_Soil_Group.html |
| Replacement cost, PR (optional) | Decimal | Average cost of a stormwater retention device (e.g. retention pond), in $/m3. Available from national or regional studies (e.g. EPA report for the US: https://www3.epa.gov/npdes/pubs/usw_d.pdf) Representative value of $1.59 USD/m3 from Simpson and McPherson (2007). |

## Summary of outputs

| Name | Type | Description |
| --- | --- | --- |
| Stormwater retention | Raster | Value of stormwater retention R |
| Pollutant load | Raster | Value of pollutant load (kg/yr) |
| Aggregated model results (optional, if watershed/ sewershed shapefile provided as input) | Shapefile | Per watershed or sewershed: - Average stormwater retention R (-) [Eq. 2-1] - Total retention volume, VR (m3/yr) [Eq. 2-2] - Total load retained (kg/yr) [Eq. 2-5] - (Optional) Total potential recharge volume, Vinf (m3/yr) [Eq. 2-3] - (Optional) Total Replacement Cost ($) [Eq. 2-6] |

## Input Guidance

Runoff coefficients

Runoff coefficients for each LULC type may not be known from previous studies. We propose a runoff coefficient calculator that requires as inputs runoff coefficients for 5 general land cover (LC) classes (see table 2 below). Such runoff coefficients can be obtained from:

- the EPA stormwater runoff calculator in the US (https://swcweb.epa.gov/stormwatercalculator/);

- any (monthly or daily time scale) rainfall-runoff model that calculates stormwater runoff and actual evapotranspiration (in mm/yr) for general LC classes (e.g. SWMM software)

- the monthly model developed by Guswa et al. (2018). The model requires monthly precipitation and ET values for a representative site in the landscape, as well as CN values for the SCS-Curve Number method (NRCS-USDA 2004).

Note that runoff coefficients for pervious LCs and bare soil should be defined for each soil hydrologic group (HSG A, B, C, D; see Table 2 for illustration). RC for water is set to 1.

After populating the RC table (and optionally IR table), the user needs to categorize each LULC (Table 1) as one (or a combination) of the general LC classes in the column “SW_Type” (Table 2). For example, the land use classes “scrub/shrub”, “grassland”, and “pasture/hay” are assigned the runoff coefficients for “pervious without tree canopy” (SW_Type=3). Some land use classes such as the “developed” categories can be assigned a combination of LC classes, and the model will compute the area-weighted average of the LC classes’ values. As an example, the “high-intensity urban” NLCD class (US classification; Table 1) represents urban areas with 80 - 100% total impervious area (nominal value 90%): it is assigned a retention coefficient that is weighted 90% “impervious without canopy”, and 10% “pervious with canopy”. Infiltration ratios (IR) are assigned to land use classes using the same approach.

Table 2. Example of Runoff coefficient and infiltration ratio table with values specified by general land cover class and soil hydrologic group (for pervious and bare soil). Values derived from SWMM simulations using 10 years of hourly weather data (2008 - 2017) at Minneapolis-St. Paul Airport, MN, USA.

| SW_Type | Description | RC_A | RC_B | RC_C | RC_D | IR_A | IR_B | IR_C | IR_D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Impervious | 0.87 | 0.87 | 0.87 | 0.87 | 0 | 0 | 0 | 0 |
| 2 | Impervious w/Tree Canopy | 0.82 | 0.82 | 0.82 | 0.82 | 0 | 0 | 0 | 0 |
| 3 | Pervious | 0.00 | 0.09 | 0.16 | 0.30 | 0.110 | 0.057 | 0.028 | 0.007 |
| 4 | Pervious w/Tree Canopy | 0.00 | 0.08 | 0.15 | 0.28 | 0.110 | 0.062 | 0.032 | 0.008 |
| 5 | Bare Land | 0.00 | 0.10 | 0.19 | 0.35 | 0.110 | 0.049 | 0.021 | 0.007 |

Pollutant Event Mean Concentrations

Pollutant event mean concentrations (EMC) may be specified by the user for any pollutant of interest. Default values for TN and TP for the urban-specific NLCD land use classes can be obtained from the US National Stormwater Quality Database (bmpdatabase.org/nsqd.html), which includes data for over 7,000 samples collected from 500+ sites over the past 30 years across the U.S., as well as from some previous summaries on less-developed land uses (Lin 2004; King and Balogh. 2011). Note: Pitt et al. (2018) found that EMCs in this database were significantly affected by land use, region, and season. Note that these data are reported with generic land use classifications (e.g. “residential”, “commercial”, “industrial”) and need to be adapted to the LULC types provided by the user. Often, a subset of these data with information on total imperviousness of the monitored watersheds can be used to aggregate sites by imperviousness. TN and TP concentrations for non-urban classes can be obtained from literature summaries, e.g. Line et al. 2002, Maestre and Pitt 2005, Lin 2004, Tetra Tech 2010, and King et al. 2011; these were used along with the U.S. NSQD to determine the EMCs in Table 1.

Users are encouraged to use results from local studies or other relevant literature values as appropriate, e.g., http://dcstormwaterplan.org/wp-content/uploads/AppD_EMCs_FinalCBA_12222014.pdf).

Representing stormwater retention techniques

Individual stormwater retention techniques like biofilters, bioretention cells, or swales can be represented by a unique LULC category, with a negative runoff coefficient, corresponding to the depth of catchment runoff they capture divided by the precipitation depth on the pixel. This requires the catchment area for the techniques to be known.

# Appendix 1: Assessing the Retention Coefficient Adjustment

Rationale: A primary concern with a grid-based approach to runoff modeling is that when aggregating results at a watershed or study site-scale, the runoff and retention loads are calculated as the sum of loads generated on every pixel – i.e. the runoff generated on each pixel is assumed to enter the drainage network of the watershed, with no chance to be retained as it moves through the network. This is a fair assumption in highly developed areas, where flow path length (i.e., distance surface runoff travels before entering a storm drain) is likely not greater than the size of the pixels (30m in U.S. NLCD/C-CAP). This was also the assumption inherent in the SWMM model as implemented to estimate runoff coefficients (Table 2), in which all runoff was routed directly to the outlet. However, in areas with substantial greenspace such as parks, cemeteries, and golf courses, and potentially outside the urban core where residential development might be less dense, “direct connection” of all constituent grid cells would lead to over-predicted loads and volumes, as additional runoff retention could be provided by infiltration in pervious areas located between pervious pixels and the storm drain network.

Further, the lack of routing also prevents any context analysis in the stormwater model; runoff being generated on a pixel (or a collection of pixels making up a parcel of interest, such as a golf course in the case of our work) is not affected by its surrounding land, nor does it have any effect on its downstream or neighboring pixels. The configuration or location of land uses within the watershed of interest have no bearing on the output, only the total amount of each land use.

Discharge data for 18 watersheds located across the metropolitan area of Minneapolis-St. Paul MN, USA (“Twin Cities” Metro Area, or TCMA) were used for testing the Runoff Retention model. These data were collected by a number of state agencies, and were publicly available. The sites could be roughly categorized by the flow regime and type of system being monitored:

- Large storm drains monitored by several watershed management organizations (Mississippi Watershed Management Organization, www.mwmo.org; Capitol Region Watershed District, www.capitolregionwd.org/monitoring-research/data/; South Washington Watershed District, wq.swwdmn.org), in which discharge was monitored annually, and for which mean annual stormflow volumes had already been determined [n=10 sites, plus 1 stream site monitored as part of stormwater permitting];

- Stream gauging sites, monitored by the Metropolitan Council Environmental Services (https://eims.metc.state.mn.us) and maintained by several local watershed districts, in which annual total (baseflow + stormflow) discharge were determined for periods of 10+ years [n = 6 sites].

For the stream gauging sites (Group 2), in which year-round monitoring has been done for 6-30 years (depending on site/constituent), data are generally of high quality, and drainage areas are known. However, the flow volumes include baseflow, which does not allow for direct comparison to Runoff Retention model, though the sites were still tested as a case study. Only the past 10 years of data were included so that the land use classification used to run the Runoff Retention model (U.S. NLCD, derived in 2013) was roughly contemporary with the gauging data; some of the watersheds have undergone substantial development over the previous 20-30 years.

Input data included 30-m U.S. NLCD land cover classification, HSG from the NRCS-USDA Soil Survey, road lines from the state of Minnesota (gisdata.mn.gov), drainage delineations and rainfall from Metropolitan Council and respective watershed districts, with additional rainfall data from Minneapolis-St. Paul Airport (retrieved from Midwest Regional Climate Center, mrcc.illinois.edu).

Results: Results of application of the Runoff Retention model to the 18 TCMA gauging sites, both with and without the retention adjustment (Eq. 2-4), are shown in Figure 1 below. Overall, the base version of the Runoff Retention model tended to over-predict observed runoff volumes for both streams and storm drain sites. Accuracy in simulation of runoff volumes was greatly improved overall when using the retention adjustment, though this was driven primarily by improvements for the storm drain sites. As these sites were generally more urban (developed), the adjusted retention appears to be an effective method to improve simulation of relatively complex connectedness in urban watersheds -- a primary purpose of the development of the Runoff Retention model as an alternative to the NDR model.

In less developed watersheds (i.e. the streams sites), it was anticipated that under-prediction of retention (over-prediction of runoff) might have resulted from the assumption of direct connection of roadways; instead, the model seems to have over-predicted retention (under-predicted runoff) in the rural watersheds. Two factors may have led to this issue: (1) stream data included baseflow, which is not predicted by the Runoff Retention model, so the simulated volumes are expected to be less than the observed volumes; and (2) the presence of drain tile in agricultural (or golf course) land use might cause some pervious land cover to be more “directly connected” than the coarse retention adjustment would predict.

|  | All Sites | Storm Drains | Streams |
| --- | --- | --- | --- |
|  |  | (Group 1) | (Group 2) |
| Root-Mean Squared Error (RMSE) |  |  |  |
| Base Model | 7.1 cm | 7.6 cm | 7.8 cm |
| With Adjusted Retention | 3.9 cm | 3.9 cm | 7.8 cm |
| Mean Absolute Error (MAE) |  |  |  |
| Base Model | 5.6 cm | 6.9 cm | 6.9 cm |
| With Adjusted Retention | 3.2 cm | 3.2 cm | 5.8 cm |

Figure 1. Upper Left: Comparison of Modeled vs. Observed Water Yield (cm) for Twin Cities Metro Area stream and storm drain sites using the default retention coefficients; Upper Right: Comparison of Modeled vs. Observed Water Yield (cm) for Twin Cities Metro Area stream and storm drain sites using the adjusted retention coefficient (Eq. 2-4); Lower Table shows RMSE and MAE parameters for the base and adjusted models, overall and as a function of site type (Streams vs. Storm Drains).

# Appendix 2: Differences between InVEST and other models

In contrast to the existing InVEST Water Yield and Nutrient Delivery Ratio models, the proposed runoff retention model is concerned primarily with surface runoff, rather than total runoff (surface and sub-surface), and designed to be implemented in urban and developing watersheds. The model uses widely available satellite-derived raster datasets, such as land cover and elevation, along with user inputs in the form of target sub-watersheds or jurisdictional boundaries for aggregation of metrics (spatial data) and, optionally, location-specific runoff and water quality parameters (tabular data). In this respect, the model is very similar to other tools, including iTree and OpenNSPECT.

OpenNSPECT (Open-source Nonpoint Source Pollution and Erosion Comparison Tool; https://coast.noaa.gov/digitalcoast/tools/opennspect.html) is a water quality scenario tool developed in 2014 by the U.S. National Oceanic and Atmospheric Administration (NOAA).

It was designed to rapidly assess scenarios of land use and climate change impacts to water, nutrient, and sediment loading in developing watersheds. Inputs are primarily in raster format, and include C-CAP or NLCD land cover (30-m resolution), elevation (up to 1m resolution), and soil hydrologic group (USDA soil surveys), as well as event- or annual-scale precipitation (gridded or station-based). Runoff is generated on each pixel using the SCS Curve Number method, taking into account land cover and soil type (hydrologic group) and including a modification for annual runoff. Mass of nutrients (load) exported from each pixel is determined as the product of this runoff volume and a mean nutrient runoff concentration (nitrogen or phosphorus) characteristic of the pixel’s land cover type. A flow direction raster is derived from the elevation data, and used to produce flow paths and drainage basin delineations over which runoff volumes and nutrient loads are routed and aggregated.

The general approach to modeling runoff and water quality in the proposed model is nearly identical to OpenNSPECT, with the following differences:

- Runoff is generated on each pixel based on runoff coefficients (runoff depth divided by rainfall depth) rather than curve number. Runoff coefficients are a function of land cover and soil hydrologic group, and are prescribed by the model but can be modified by the user based on output of other models (e.g. SWMM), local hydrology data, modified curve numbers, etc.;

- The model estimates potential groundwater recharge through use of an infiltration ratio parameter, which is also prescribed by the model based on SWMM simulations in test watersheds but can be modified by the user.

Note: add comparison between InVEST and iTree (copy from SF working paper)

Additional resources for further hydrologic studies:

- LTER model: https://www.sciencedirect.com/science/article/pii/S0301479717301111

  - https://swtelr.2nform.com/

References:

Arkema, K. K., Griffin, R., Maldonado, S., Silver, J., Suckale, J., & Guerry, A. D. (2017). Linking social , ecological , and physical science to advance natural and nature-based protection for coastal communities. https://doi.org/10.1111/nyas.13322

Balbi, M., Lallemant, D., & Hamel, P. (2017). A flood risk framework for ecosystem services valuation: a proof-of-concept.

King, K.W. and Balogh, J. (2011). Stream water nutrient enrichment in a mixed-use watershed. J. Environ. Monit, 13: 721-731.

Lin, J.P. (2004). Review of published export coefficient and event mean concentration (emc) data. Wetlands Regulatory Assistance Program. ERDC TN-WRAP-04-3. Sep 2004.

Line, D.E., White, N.M., Osmond, D.L., Jennings, G.D. and Mojonnier, C.B. (2002). Water Environment Research, 74(1): 100-110.

Maestre, A. and Pitt, R. (2005). The National Stormwater Quality Database, Version 1.1: A Compilation and Analysis of NPDES Stormwater Monitoring Information. Center for Watershed Protection; Ellicott City, MD. Sep 4, 2005.

NRCS-USDA. (2004). Chapter 10. Estimation of Direct Runoff from Storm Rainfall. In United States Department of Agriculture (Ed.), Part 630 Hydrology. National Engineering Handbook. Retrieved from http://www.nrcs.usda.gov/wps/portal/nrcs/detailfull/national/water/?cid=stelprdb1043063

Pitt, R., Maestre, A. & Clary, J. (2018). The National Stormwater Quality Database (NSQD), Ver 4.02. Retrieved from http://www.bmpdatabase.org/Docs/NSQD_ver_4_brief_Feb_18_2018.pdf

Sahl, J. (2015). Economic Valuation Approaches for Ecosystem Services: a literature review to support the development of a modeling framework for valuing urban stormwater management services.

Simpson, J.R. and McPherson, E.G. (2007). San Francisco Bay Area State of the Urban Forest Final Report. Center for Urban Forest Research, USDA Forest Service Pacific Southwest Research Station. Davis, CA. Dec 2007: 92 pp.

Tetra Tech, Inc. (2010). Stormwater Best Management Practices (BMP) Performance Analysis. Prepared for U.S. E.P.A. Region 1. Fairfax, VA. 232 pp.